# Summary

This document is designed to inform users about the usage of the 4Evac Maze Manager application. This document is written for people that are familiar with 4Evac devices. The general workflow will be discussed first and afterwards the different application screens will be described.

The application works on Windows 7 and up. Older versions and other platforms are not supported to date.

USER MANUAL

| Rev. | Date | Nature of Changes | Approved By |
| --- | --- | --- | --- |
| 1 | 04-07-2022 | Created 4Evac Maze Manager | TC |

www.4Evac.com

Table of Contents:

Summary	1

1. Initial Configuration	3

1.1.	Windows Firewall	3

1.2.	Maze Module IP Range	3

2.	General Application Overview	4

3.	Maze Module IP Settings	6

4.	Maze Module Settings	6

5.	Device Settings	6

# Initial Configuration

When this application is used there are a couple of rules to follow to make it function correctly.

- Make sure the application is allowed through the firewall.

- Make sure the selected Ethernet Interface is in the same IP range as the Maze modules.

- Make sure that only the network adapter that is used for communication with the Maze modules is enabled.

## Windows Firewall

When the application is started for the first time and a ethernet interface is selected. Windows will ask for permission for the application to go through the firewall. Make sure you allow this, otherwise the application will not find any Maze modules.

See screenshot below for Windows Defender Firewall settings:

Figure 1 Windows Defender Firewall Settings

## Maze Module IP Range

New Maze modules come with dynamic IP address selection enabled. This normally uses the following IP range: 169.254.159.x

The Maze modules should show up in this application even when the selected ethernet interface has another IP range configured. But in case this does not work try to set the ethernet interface to a static IP address in the standard dynamic IP range.

# General Application Overview

The general overview of the application consists of 2 important parts.

The first part is the connection settings. Here the user can select the ethernet interface that is used for communication. If the initial configuration is followed, there should only be 1 interface that can be selected. Once the correct ethernet interface is selected with the “Apply” button, it will be possible to use the rest of the functionality. This starts with rescanning the connected devices with the ”Rescan” button or stopping the scanning with the “Stop” button.

Note: After the “Scan / Rescan” button is pressed it can take 5-10 seconds before modules will show up in the table.

It is also possible to verify the module settings with the “Verify Module Settings” button. This scans all connected modules and makes the following assumptions:

- All devices with a module id between 1 and 255 are impact devices. This means that only 1 maze channel can be configured for talking.

- All devices with a module id between 256 and 2000 are DCA amplifiers. This means the user cannot configure the settings for the first maze channel. The second maze channel is used to configure to what maze audio channel the BGM input on the DCA is send.

- All devices with a module id above 2000 are general devices where no settings are checked. This means the user can configure all available 32 maze channels.

The “Standalone System” checkbox disables the verification of settings. It also removes any restriction for editing Maze module settings. This should only be used when the system is used as standalone application, so without any 4Evac devices connected to the system.

The “Export Settings” button generates a .csv file with the module ids, the IP address, the module name and the configured audio channels. This file can be imported in the 4Evac Manager “Sources” tab.

The second part is the device table, this will show all connected Maze devices. For each connected module there are 3 buttons available. The first button is to edit the Maze module settings itself. This includes the module id, name and maze audio channel configuration. The Second button allows for adjusting the Maze module IP settings. The last button allows for changing device specific settings. This is only supported for the DCA and the Audio Matrix.

Note: When the “Edit Device Settings” button is grayed out, it could mean that the IP address is in a different range, which stops the user from editing these settings. It could also be that the maze module id was changed and the device was not power cycled yet.

Note: When the “Edit…” text is shown in red. It means that there is a conflict with engine id settings.

Figure 2 General Application Overview

# Maze Module IP Settings

The Maze module supports 2 IP modes. Dynamic addressing or static. These settings can be changed in this dialog. After pressing “Ok” will the IP settings be changed and the module list will be rescanned.

Figure 3 Ip Address Configuration

# Maze Module Settings

Each maze module has some general settings and some routing settings. The general settings are described below and the routing settings are explained after that per group.

Note: All changes are applied right away on selection. Volume and routing changes can be seen and heard directly in the system. The user can only restore previous settings by power cycling a device as long as the “Save Settings Permanently” button is not used.

The general settings include selecting the module name. This name will be visible in the device table in the main screen. This is only used for indication during configuration. It also has 2 buttons at the bottom of the dialog. The “Refresh Settings” will retrieve all settings again from the module. This can be used for example to see how the 4Evac system changes settings in real time during usage.

For each engine the following settings are available:

- The module id can be selected. When the modules are used in combination with the 4Evac system, the engine 1 ID and the engine 2 ID should have the same value. The module id is used to select what kind of device is connected and which settings will be shown for the routing. The 3 groups are:

  - All devices with a module id between 1 and 255 are impact devices.

  - All devices with a module id between 256 and 2000 are DCA amplifiers.

  - All devices with a module id above 2000 are audio matrix devices.

- The general audio volume from -60dB to +6dB.

- The sidetone from -60dB to + 6dB. This setting is used to direct any audio being send to the maze network to also play with some volume on the local output. This extra playback is disabled when the sidetone is set to -60dB.

- The threshold at which the audio from any of the talk channels is send to the network. This is a noise gate to stop the module from sending audio data when it’s not needed. This can be set from -60dB to 0db.

Figure 4 Maze Module General Settings

## Impact Maze Device Settings

The maze impact device only has 1 maze channel available per engine. Depending on how the impact is used, the user should configure different settings.

The first option is when the impact is used in combination with maze amplifiers. In this case the user should:

- Select 2 maze audio channel ids, one for each engine. These audio channel ids should be unique over all used maze modules. Since they are used to identify what channels the impact can use to stream audio to its maze amplifiers.

- Set the talk mode to enabled on both engines. This allowed the impact to send audio to the Maze network.

- Set the volume of each input. -60dB to +6dB.

The second option is when the impact is using normal DCA amplifiers and is only using the BGM inputs to send audio data to the Maze network. Engine 1 settings should be used for BGM1 and engine 2 settings should be used for BGM2. In this case the user should:

- Select a maze audio channel id for each BGM input that is used. The audio channel ids should be unique over all used maze modules.

- Set the talk mode to enabled. This allowed the impact to send audio to the Maze network.

- Set the volume of each input. -60dB to +6dB.

Figure 5 Impact Maze Settings

## DCA Maze Device Settings

The DCA has 2 maze channels available per engine. The first channel is not user configurable, since this is reserved for controlling audio channels the DCA should listen to via its impact master. When the system is operated and the “Refresh Settings” button is clicked, it can show to what audio channel a DCA is switched in the first channel.

The user can configure the second channel when the BGM inputs on the DCA are used. Engine 1 settings should be used for BGM1 and engine 2 settings should be used for BGM2. In this case the user should:

- Select a maze audio channel id for each BGM input that is used. The audio channel ids should be unique over all used maze modules.

- Set the talk mode to enabled. This allowed the impact to send audio to the Maze network.

- Set the volume of each input. -60dB to +6dB.

Figure 6 DCA Maze Settings

## Audio Matrix Device Settings

The Audio Matrix has 32 maze channels available per engine. For each channel the user can:

- Select a maze audio channel id, The audio channel ids should be unique over all used maze modules. At least when you don’t want audio mixing together when the channel is set as talk mode.

- Set the talk mode. This allowed the audio matrix to send audio to the Maze network.

- Set the listen mode. This allowed the audio matrix to listen to audio from the Maze network.

- Set the volume of each channel. -60dB to +6dB.

Figure 7 Audio Matrix Settings

# Device Settings

The DCA and the audio matrix have some extra user configurable settings that can be adjusted via this application. This can be used for some extra input and output filtering on the device itself. It can also be used to change the internal routing of the device.

Note: When the routing is changed from the default settings it can stop the 4Evac devices from working as expected.

## DCA Settings

The DCA starts with an overview of parts that can be adjusted. There are 3 different parts. The first are the inputs, followed by the routing matrix and lastly the output processing. The main screen has a button to “Save Settings Permanently”, if not saved all settings are lost after the device is power cycled.

Figure 8 DCA Settings Overview

### DCA Input Settings

The input has 2 tabs with settings. In the first tab the volume input can be selected. From -12dB to +64dB. The second tab allows selection of equalizer settings.

Figure 9 DCA Input Volume Settings

Figure 10 DCA Input Equalizer Settings

### DCA Mixer Settings

The mixer allows the user to change the routing of the inputs to the outputs. The default settings for the DCA are: that audio from the maze engine is routed to the output of the maze engine. Nothing is directly routed from the DCA input to the output.

Figure 11 DCA Mixer Settings

### DCA Output Settings

The output has 2 tabs with settings. In the first tab the equalizer settings can be adjusted. The second tab allows selection of the following settings:

- Correction of aux volume from -63dB to +9dB.

- Correction of amplifier volume from -63dB to +9dB.

- Amplifier 20kHz level from -63dB to -20dB.

- Output Delay from 0 to 10000 ms.

Figure 12 DCA Output Equalizer Settings

Figure 13 DCA Output More Settings

## Audio Matrix Settings

The audio matrix starts with an overview of parts that can be adjusted. There are 3 different parts. The first are the inputs, followed by the routing matrix and lastly the output processing. The main screen has a button to “Save Settings Permanently”, if not saved all settings are lost after the device is power cycled.

Figure 14 Audio Matrix Settings Overview

### Audio Matrix Input Settings

The input has 4 tabs with settings.

In the first tab the input gain can be adjusted from -64dB to 0 dB.

Figure 15 Audio Matrix Input Settings

The second tab allows changing of the following gate settings:

- Select the control channel. Line input 1 - 4 or Analog input 1 – 4.

- Select the threshold from -60dB to 0dB.

- Select the hold time from 0 to 30000 ms.

- Select the response time from 0 to 1000 ms.

Figure 16 Audio Matrix Input Gate Settings

The third tab allows changing of the following dynamics settings:

- Select the control channel. Line input 1 – 4.

- Select the threshold from -60dB to 0dB.

- Select the ratio from 20:1 to 1:20.

- Select the attack from 0 to 1000 ms.

- Select the release from 0 to 10000 ms.

Figure 17 Audio Matrix Input Dynamics Settings

The last tab allows changing of the equalizer settings.

Figure 18 Audio Matrix Input Equalizer Settings

### Audio Matrix Mixer Settings

The mixer allows the user to change the routing of the inputs to the outputs. The default settings for the audio matrix are: nothing is routed from any input to any output.

Figure 19 Audio Matrix Mixer Settings

### Audio Matrix Output Settings

The first tab allows changing of the following dynamics settings:

- Select the control channel. Line input 1 – 4 or Analog input 1 - 4.

- Select the threshold from -60dB to 0dB.

- Select the ratio from 20:1 to 1:20.

- Select the attack from 0 to 1000 ms.

- Select the release from 0 to 10000 ms.

Figure 20 Audio Matrix Output Dynamics Settings

The second tab allows changing of the equalizer settings.

Figure 21 Audio Matrix Output Equalizer Settings

The last tab allows changing of the following output settings:

- Correction of the output volume from -64dB to +24dB.

- Delay of the line from 0 to 10000 ms.

Figure 22 Audio Matrix Output Settings